LLD-01-002 — Network Segmentation

Project Redstone | Northwind Outdoor Ltd. | Design Process Working Paper

| Document control | Details |
| --- | --- |
| Document ID | LLD-01-002 |
| Design question | Q02 — What network segments are required for the first Northwind implementation, and why does each one exist? |
| Revision | v1.1 — Proposed amendment |
| Status | Draft for Technical Review |
| Design area | LLD-01 — Platform & Infrastructure Foundation |
| Related designs | LLD-02, LLD-06, LLD-07, LLD-09, LLD-12 |
| Change purpose | Reconcile the original conceptual network segmentation design with the subsequently approved VLAN, firewall, routing and administrative access architecture |

1. Decision Summary

Northwind v1 will implement a segmented network architecture based on security trust boundaries and communication requirements, rather than separating systems solely according to their technical functions.

The original design identified five primary security zones.

| Security zone | Purpose | Current implementation |
| --- | --- | --- |
| USER | Employee endpoints and normal business activity | VLAN 300 |
| SERVER | Internal infrastructure and business services | VLAN 200 |
| MGMT | Privileged infrastructure administration | VLAN 100 |
| PERIPHERAL | Printers and other lower-trust embedded/peripheral devices | VLAN 400 |
| EXTERNAL / UNTRUSTED | Internet/WAN and networks outside Northwind's internal trust boundary | External connectivity through FW01; no Northwind internal VLAN |

The original five-zone model remains valid.

Subsequent LLD-06 design decisions have established the four internal VLANs, their proposed addressing scheme and the intended network connectivity architecture.

Traffic between internal security zones shall follow a default-deny, explicitly permitted firewall policy.

Communication shall be authorised according to documented business, service and administrative requirements.

FW01 shall provide the principal routed security boundary, while JUMP01 shall provide the controlled administrative entry point to internal RHEL servers.

The implementation shall also retain independent management access to the Proxmox hypervisor, HYP01, to support bootstrap and emergency recovery.

1.1 Current design status

The conceptual security-zone model is approved.

The VLAN identifiers, internal network architecture and administrative access model have been established through subsequent detailed design work.

Individual firewall rules, final host IP addresses, DHCP scopes and some service-specific communication requirements remain subject to detailed specification and implementation validation.

2. Design Principles

The original design established several principles that remain applicable.

2.1 Segmentation represents trust boundaries

Network segments should not be created simply because systems perform different functions.

A separate network is justified where systems have materially different:

- Security characteristics.

- Exposure requirements.

- Administrative requirements.

- Communication requirements.

- Patching or lifecycle characteristics.

- Compromise impact.

This approach avoids unnecessary VLAN proliferation while maintaining meaningful security boundaries.

The four internal VLANs established through LLD-06 reflect these distinctions.

2.2 Inter-zone communication follows least privilege

Placement within a network zone does not automatically grant unrestricted access to another zone.

Traffic crossing security boundaries must be explicitly permitted according to:

Source → Destination → Service → Justification

Illustrative service requirements include:

| Source | Destination | Service | Intended policy |
| --- | --- | --- | --- |
| USER | CORE01 | DNS | Allow where required |
| USER | FILE01 | SMB | Allow where required |
| USER | FILE01 | SSH | Deny |
| USER | Internal SERVER subnet | Unapproved services | Deny |
| JUMP01 | Authorised RHEL server | SSH | Allow |
| Unapproved home-network device | JUMP01 | SSH | Deny |

These examples express intended policy, not a complete or implemented firewall rule set.

The final connectivity matrix and firewall rules shall be maintained through LLD-06 and the relevant service designs.

2.3 Service consumption and service administration are separate

Normal users require access to business services but do not require administrative access to the infrastructure providing those services.

For example:

USER → FILE01 → SMB

represents consumption of the file-sharing service.

Administrative access follows a separate path:

Approved Ubuntu Workstation → FW01 → JUMP01 → FILE01

The destination server must independently authenticate and authorise the administrator.

This separation reduces unnecessary exposure of administrative interfaces and protocols.

2.4 Network segmentation is not sufficient on its own

VLANs establish logical network separation, but they do not independently provide a complete security model.

Security controls must also include:

- Stateful firewall policies at routed security boundaries.

- Host-level firewall rules.

- Secure administrative authentication.

- Least-privilege authorisation.

- Operating system hardening.

- Logging and validation.

In particular, traffic between systems within the same VLAN may not traverse FW01.

RHEL servers shall therefore use firewalld to enforce appropriate host-level access restrictions.

3. USER Zone

3.1 Purpose

The USER Zone provides network connectivity for normal Northwind employee endpoints and day-to-day business activities.

3.2 Intended network

| Property | Design |
| --- | --- |
| Security zone | USER |
| VLAN | 300 |
| Proposed subnet | 10.20.30.0/24 |
| Proposed gateway | 10.20.30.1 on FW01 |
| Typical endpoints | Employee desktops, laptops and managed user devices |
| DHCP | Planned through CORE01, with relay functionality provided by FW01 |

The subnet and gateway values are the current LLD-06 design proposal and remain subject to final network configuration confirmation.

3.3 Expected systems

Examples include:

- Office desktops.

- Employee laptops.

- Managed staff wireless clients.

- Authorised administrator workstations when performing ordinary user activities.

Printers are deliberately excluded and assigned to the PERIPHERAL Zone.

The initial Ubuntu administration laptop is a special case: it resides on the physical home network and uses an approved routed administrative path through FW01. It is not required to join VLAN 300 to perform administration.

3.4 Rationale

Employee endpoints represent a different trust class from Northwind infrastructure.

User devices routinely interact with:

- Email.

- Web browsers.

- Downloaded files.

- External websites.

- User-supplied data.

- Potentially removable media.

They therefore represent a comparatively exposed part of the environment.

Separating employee endpoints from servers and management infrastructure limits the potential impact of a compromised workstation and supports defence-in-depth.

3.5 Communication principles

USER systems shall be permitted to access only the infrastructure and business services required for normal operation.

Examples include DNS, approved file sharing and other explicitly authorised services.

General SSH access from USER to internal servers shall not be permitted.

3.6 Remote users

The original requirement for remote and office users to receive a broadly consistent service experience remains applicable.

However, the placement of future remote-access clients is deferred.

VPN clients may eventually be assigned to the USER Zone or to a dedicated remote-access network with equivalent approved access permissions.

No dedicated VPN client network is required for the initial FW01/JUMP01 implementation.

4. SERVER Zone

4.1 Purpose

The SERVER Zone contains internal infrastructure and business services consumed by Northwind systems and employees.

4.2 Intended network

| Property | Design |
| --- | --- |
| Security zone | SERVER |
| VLAN | 200 |
| Proposed subnet | 10.20.20.0/24 |
| Proposed gateway | 10.20.20.1 on FW01 |
| Typical systems | CORE01, OPS01, BACKUP01, FILE01 |
| Security control | FW01 inter-zone policy and individual RHEL host firewalls |

The listed server placements are the current architectural direction; individual VM interface assignments remain subject to final specifications.

4.3 Intended services

The current six-VM infrastructure plan includes the following SERVER workloads.

| VM | Primary services |
| --- | --- |
| CORE01 | DNS, DHCP and NTP/time services |
| OPS01 | Monitoring and logging |
| BACKUP01 | Backup management and recovery services |
| FILE01 | Business file sharing |

The original document also identified potential identity and application services.

A dedicated identity server is not currently required for Northwind v1 because the initial design uses Linux-native local accounts and access controls.

Future application services shall be introduced according to approved business requirements.

4.4 Rationale

Servers have different security and operational requirements from employee endpoints.

Separating servers from USER systems allows Northwind to control which services users may consume without exposing unnecessary server interfaces.

For example:

USER → FILE01 → SMB — permitted when required.

USER → FILE01 → SSH — prohibited for routine user access.

The firewall boundary between USER and SERVER provides a mechanism for enforcing this distinction.

4.5 Application segmentation

Application servers shall not automatically receive dedicated network segments.

Additional segmentation should only be introduced where an application has materially different exposure, sensitivity or communication requirements.

A future architecture containing an Internet-facing web server, application backend and sensitive database could justify additional security zones.

No such requirement has been established for the initial Northwind infrastructure.

5. MGMT — Management Zone

5.1 Purpose

The Management Zone provides a trusted network path for privileged administration of Northwind infrastructure.

It is not limited to the management of network equipment.

Management activities may include:

- Linux server administration.

- Firewall administration.

- Monitoring administration.

- Infrastructure configuration.

- Security maintenance.

- Recovery operations.

5.2 Intended network

| Property | Design |
| --- | --- |
| Security zone | MGMT |
| VLAN | 100 |
| Proposed subnet | 10.20.10.0/24 |
| Proposed gateway | 10.20.10.1 on FW01 |
| Initial principal VM | JUMP01 |
| Access model | Restricted SSH through FW01 |
| Administrative workstation | Approved Ubuntu laptop on the external home network |

5.3 Important distinction: HYP01 management

The original conceptual design included Proxmox management within the Management Zone.

Subsequent infrastructure and administrative access design established an important implementation distinction.

HYP01's Proxmox management interface shall remain independently accessible through the physical home network, rather than depending upon FW01 or JUMP01.

This supports:

- Initial FW01 installation and bootstrap.

- Initial JUMP01 installation.

- Recovery when internal networking fails.

- Proxmox VM console access.

- Emergency restoration of administrative connectivity.

HYP01 therefore forms part of Northwind's privileged management trust model, but its management interface is not dependent on VLAN 100.

This is a deliberate recovery and bootstrap requirement, not permission for unrestricted access from the home network.

HYP01 management access must be protected separately.

5.4 Rationale

Administrative traffic has fundamentally different trust requirements from ordinary business traffic.

Separating management access reduces the number of systems capable of reaching privileged interfaces and provides clearer boundaries for:

- Access control.

- Administrative logging.

- Auditing.

- Monitoring.

- Privileged access management.

5.5 Administrative access model

Northwind shall use JUMP01 as the standard administrative entry point for internal RHEL servers.

Approved Ubuntu Administration Laptop

Physical home network • SSH private keys remain here

Explicit route and restricted SSH

FW01

Network security boundary • Source and destination restrictions

JUMP01 — MGMT VLAN 100

Single NIC • SSH ProxyJump • Restricted forwarding

Authorised Internal RHEL Server

Independent SSH authentication and sudo authorisation

Independent recovery path

Ubuntu/home network → HYP01 Proxmox management → VM console

JUMP01 shall use a single network interface connected to the MGMT VLAN.

It shall not be dual-homed or connected to multiple VLANs for routine administration.

Inter-VLAN routing remains the responsibility of FW01.

5.6 Why a bastion was selected

The original design considered three approaches:

- Dual-homed or trunk-connected administrator workstations.

- Direct administrative access from selected USER workstations.

- A dedicated bastion/jump host.

Dual-homed administrator workstations were considered technically feasible but would connect a potentially exposed endpoint directly to the trusted management environment.

Direct administrative exceptions would be simpler initially but could require multiple endpoints to hold privileged network access.

The bastion approach was selected because it establishes a consistent, controlled administrative path.

Subsequent LLD-07 decisions further defined this model through:

- Approved administrative workstations.

- Named local administrator accounts.

- SSH public-key authentication.

- SSH ProxyJump.

- Restricted forwarding.

- sudo-based privilege management.

- Documented access revocation.

5.7 Availability consideration

JUMP01 becomes operationally important because its failure may prevent normal administrative access to internal RHEL servers.

However, Northwind's recovery-based resilience strategy does not currently justify a redundant bastion.

Instead, emergency recovery shall use the independently accessible HYP01 Proxmox console and documented recovery procedures.

6. PERIPHERAL Zone

6.1 Purpose

The PERIPHERAL Zone isolates network-connected printers and other lower-trust embedded devices from managed employee endpoints and infrastructure.

6.2 Intended network

| Property | Design |
| --- | --- |
| Security zone | PERIPHERAL |
| VLAN | 400 |
| Proposed subnet | 10.20.40.0/24 |
| Proposed gateway | 10.20.40.1 on FW01 |
| Typical devices | Network printers and multifunction printers/scanners |

The existence of VLAN 400 in the design does not imply that physical printers have already been connected or commissioned.

6.3 Rationale

Printers represent a different security class from managed employee workstations.

Such devices may have:

- Vendor-specific firmware.

- Different patching mechanisms.

- Long operational lifetimes.

- Limited endpoint security capabilities.

- Embedded web-management services.

- Protocols such as IPP, SNMP or SMB.

- Stored or cached business information.

Segmentation compensates partially for the reduced endpoint security control available on embedded devices.

6.4 Communication principles

| Source | Destination | Intended policy |
| --- | --- | --- |
| USER | PERIPHERAL | Allow required printing services |
| PERIPHERAL | USER | Deny unsolicited connections |
| PERIPHERAL | SERVER | Deny by default |
| PERIPHERAL | MGMT | Deny by default |
| JUMP01 / authorised administration | PERIPHERAL | Allow approved management where required |
| PERIPHERAL | Internet | Restrict to individually justified services |

Specific exceptions may be necessary for DNS, NTP, mail relay or approved file services.

Such flows must be justified individually.

7. EXTERNAL / UNTRUSTED Zone

7.1 Purpose

The External/Untrusted Zone represents networks outside Northwind's trusted internal infrastructure.

In the laboratory implementation, this includes the physical home network and its upstream Internet connectivity.

The home network is not equivalent to a dedicated enterprise WAN or an isolated production Internet connection.

7.2 FW01 external connectivity

FW01 shall connect to the physical home network through Proxmox vmbr0.

The home router provides upstream Internet connectivity.

FW01 shall provide outbound NAT for Northwind internal networks, resulting in a double-NAT arrangement for Internet-bound traffic where the home router also performs NAT.

7.3 Outbound communication

Northwind servers may require Internet connectivity for:

- Operating system updates.

- Official package repositories.

- Approved software downloads.

- External business services.

- Temporary external time services during bootstrap.

The ability to initiate an outbound connection does not imply that external systems should be permitted to initiate connections toward internal servers.

Stateful firewalling shall enforce this distinction.

7.4 Home-network access restrictions

Northwind internal systems shall not receive general access to devices on the physical home network.

The approved administrative path from the Ubuntu laptop to JUMP01 is a specific exception governed by FW01 policy.

Similarly, HYP01 management access is a separately protected administrative function.

7.5 Future public services

If Northwind later introduces Internet-facing applications or APIs, those systems shall not automatically be placed within the internal SERVER Zone.

Such requirements may justify a DMZ or equivalent security boundary.

No current initial implementation requirement justifies introducing a DMZ.

8. Proxmox Virtual Networking Architecture

Subsequent LLD-06 design decisions established the intended Proxmox networking model.

8.1 Virtual bridges

| Bridge | Purpose | Design characteristics |
| --- | --- | --- |
| vmbr0 | External/home network connectivity | Physical uplink; HYP01 management and FW01 external interface |
| vmbr1 | Internal Northwind networking | VLAN-aware virtual bridge; no physical uplink; no HYP01 management IP |

This architecture allows multiple isolated internal networks to operate on a single physical Proxmox host without requiring a physical managed switch for the initial lab.

8.2 FW01 network interfaces

FW01 shall use two virtual network interfaces.

| Interface role | Connection | Purpose |
| --- | --- | --- |
| External | vmbr0 | Connectivity to the physical home network |
| Internal | vmbr1 | VLAN trunk supporting Northwind's internal VLANs |

FW01 shall provide routing and policy enforcement between the internal VLANs.

8.3 Guest VM connections

Ordinary RHEL guest VMs shall connect to the appropriate internal VLAN using access-tagged virtual network interfaces.

JUMP01 shall have a single NIC connected to VLAN 100.

CORE01, OPS01, BACKUP01 and FILE01 are intended to use the SERVER network, subject to their final VM specifications.

8.4 Host independence

The Proxmox host shall not depend upon FW01 for access to its management interface.

No HYP01 management address is required on vmbr1.

This prevents the failure of FW01 from removing the primary management path needed to recover FW01 itself.

9. Monitoring Considerations

Monitoring traffic and management traffic should not automatically be treated as equivalent.

A monitoring system may need to query:

- FILE01.

- CORE01.

- FW01.

- JUMP01.

- HYP01.

These represent monitoring and observability flows.

An administrator accessing the monitoring platform represents privileged management activity.

9.1 OPS01 placement

The current infrastructure plan identifies OPS01 as the monitoring and logging server.

OPS01 is intended to reside within the SERVER Zone.

Its monitoring and logging traffic shall be permitted according to documented source, destination and service requirements.

It does not require placement in MGMT solely because it monitors infrastructure.

9.2 Security considerations

Monitoring permissions shall not automatically confer administrative privileges.

Where monitoring requires access to privileged interfaces, the permissions and credentials must be explicitly controlled.

Monitoring access to HYP01 shall also respect the independent hypervisor management boundary.

10. Backup Network Consideration

A dedicated backup network was considered during the original design discussion.

Potential advantages include:

- Isolation of backup traffic.

- Reduced contention with other workloads.

- Tighter control of backup infrastructure communication.

- Additional protection of recovery services.

However, a dedicated backup network also introduces additional VLANs, interfaces, routing requirements, firewall policies and operational complexity.

10.1 Current decision

Northwind v1 does not currently justify a dedicated backup VLAN.

BACKUP01 is included within the planned SERVER infrastructure.

Backup traffic shall be controlled using the existing network segmentation and host-level security controls.

10.2 Future review

The requirement for a dedicated backup network may be reconsidered if future workload growth, backup performance or security requirements justify it.

The need for an independent backup destination outside HYP01's physical failure domain remains separate from the question of whether backup traffic requires a dedicated VLAN.

Status: Dedicated backup network deferred.

11. Application Network Consideration

Dedicated application VLANs were also considered.

The original decision remains valid: systems shall not be segmented simply because they perform different technical functions.

Additional application security zones should be introduced only where differences in exposure, sensitivity, data classification or communication requirements create a genuine trust boundary.

Northwind v1 therefore retains the general SERVER Zone.

Status: Additional application-specific segmentation deferred unless justified by subsequent service design.

12. Updated Conceptual v1 Network Architecture

The original conceptual network diagram identified the five security zones and a firewall-controlled management path.

The following logical model incorporates the subsequent LLD-06 and LLD-07 decisions.

Physical Home Network / Internet

Home router • Ubuntu admin laptop • HYP01 management

Proxmox vmbr0

FW01 — RHEL Firewall/Router

Stateful filtering • Inter-VLAN routing • NAT

Internal trunk via vmbr1

MGMT — VLAN 100

10.20.10.0/24

JUMP01

Controlled SSH access

SERVER — VLAN 200

10.20.20.0/24

CORE01 • OPS01

BACKUP01 • FILE01

USER — VLAN 300

10.20.30.0/24

Employee endpoints

PERIPHERAL — VLAN 400

10.20.40.0/24

Printers and peripherals

Conceptual architecture, not a representation of deployed interfaces or final firewall rules. FW01 mediates routed traffic between the internal VLANs.

12.1 Important architectural distinctions

The diagram should be interpreted alongside the following requirements:

- HYP01 management remains on the external/home-network side, independent of FW01.

- JUMP01 resides in VLAN 100 and uses a single virtual NIC.

- Internal servers are reached administratively through JUMP01.

- The Ubuntu administration laptop remains on the home network.

- FW01 controls permitted administrative ingress and inter-VLAN traffic.

- The physical home network is not treated as a trusted Northwind internal zone.

- Same-VLAN traffic requires host-level security controls because it may bypass FW01.

13. Security Policy Principle

The following principle governs Northwind's firewall design:

Inter-zone communication shall follow a default-deny, explicitly permitted model based upon documented business, service and administrative requirements.

Firewall permissions shall be defined according to:

Source → Destination → Required Service → Business/Technical Justification

Creation of separate VLANs without corresponding access-control policies is not considered sufficient segmentation.

13.1 FW01 policy requirements

FW01 shall:

- Restrict traffic between internal security zones.

- Permit approved service communication.

- Restrict administrative access to JUMP01.

- Prevent general direct SSH access from ordinary user endpoints to internal servers.

- Restrict unsolicited traffic from the external/home network.

- Provide controlled outbound connectivity.

- Support stateful return traffic for authorised sessions.

13.2 Defence-in-depth

Each RHEL server shall also enforce appropriate host-level firewall policies.

SELinux, SSH hardening and sudo authorisation shall provide additional security layers.

13.3 Validation

The implementation shall demonstrate both:

- Positive tests confirming required connectivity.

- Negative tests confirming prohibited connectivity.

Successful ping or SSH access alone is not sufficient evidence that the segmentation policy is secure.

14. Key Risks and Mitigations

| Risk | Mitigation / design response |
| --- | --- |
| Compromised USER endpoint attempts lateral movement | Separate USER from infrastructure and explicitly permit required SERVER services |
| Administrative interfaces exposed to ordinary endpoints | Dedicated MGMT trust boundary and JUMP01-based administration |
| Compromised administrator workstation obtains privileged network access | Approved workstation restrictions, SSH authentication, destination authorisation and least privilege |
| JUMP01 failure prevents normal administration | Recovery-based resilience and independent HYP01 console access |
| FW01 failure interrupts Northwind connectivity | Independent HYP01 management and documented firewall recovery |
| Compromised printer becomes an internal foothold | Dedicated PERIPHERAL VLAN and restricted communication |
| Excessive VLAN complexity | Introduce additional segments only for justified trust boundaries |
| Public service compromise affects internal systems | Evaluate DMZ architecture before introducing public-facing services |
| Firewall rules become overly permissive | Default-deny model, documented exceptions and negative testing |
| Segmentation blocks legitimate services | Service connectivity matrix and controlled validation |
| Same-VLAN traffic bypasses FW01 | RHEL firewalld and service-level access restrictions |
| Home-network exposure creates an unintended management path | Restrict FW01 ingress and separately protect HYP01 management |
| Overlapping subnet addressing causes routing problems | Validate home, Northwind and relevant VPN address ranges before implementation |
| Misconfigured VLANs or trunking cause isolation failure | Verify Proxmox bridge settings, VM tags, FW01 interfaces and permitted flows |

15. Deferred Detailed Design

The original working paper deferred a number of technical decisions until the server roles and network requirements were better understood.

Several have since been resolved through LLD-06 and LLD-07.

| Original deferred item | Updated status |
| --- | --- |
| VLAN IDs | Established: 100, 200, 300, 400 |
| IPv4 addressing | Proposed subnet plan established |
| IPv6 addressing | Not fully resolved; requires policy confirmation |
| Subnet sizes | Proposed /24 per internal VLAN |
| Default gateways | FW01 .1 addresses proposed |
| DHCP scopes | Detailed configuration remains to be confirmed |
| DNS configuration | CORE01 identified; detailed records and bootstrap arrangements require confirmation |
| Physical switch-port configuration | No internal physical switch required for initial virtual-only networking |
| Trunk/access-port assignments | FW01 internal trunk and guest access-tagging established conceptually |
| Detailed firewall rules | Remain subject to implementation specification |
| Exact protocol/port matrix | LLD-06 connectivity matrix requires service-level completion |
| VPN client network placement | Deferred |
| Monitoring-server network placement | OPS01 planned in SERVER |
| Backup network design | Dedicated backup VLAN deferred |
| DMZ design | Deferred |
| Application-specific segmentation | Deferred unless justified |

15.1 Remaining implementation confirmations

Before the network design is accepted for operational use, the following details require confirmation:

- Actual home-network addressing and FW01 external interface configuration.

- Overlap-free Northwind subnet assignments.

- Exact FW01 interface names and VLAN subinterface configuration.

- Final VM addresses.

- Firewall zone and rule implementation.

- Required routing and return-path behaviour.

- CORE01 service dependencies and bootstrap arrangements.

- Positive and negative security test evidence.

16. Implementation and Learning Implications

The design creates practical implementation work directly relevant to Northwind's Linux administration and RHCSA objectives.

Relevant technical areas include:

- Linux network interface configuration.

- NetworkManager and nmcli.

- VLAN interfaces and routing.

- Stateful firewall configuration.

- firewalld zones and services.

- SSH administration.

- SSH ProxyJump.

- SSH key management.

- User and group administration.

- sudo configuration.

- Service exposure and port identification.

- Logging and auditing.

- SELinux.

- DNS and DHCP services.

- Network troubleshooting.

- Firewall policy validation.

- Recovery from network and SSH configuration failures.

It also allows existing networking knowledge to be applied directly to Linux systems administration.

The project should demonstrate not only that traffic can be routed successfully, but that communication is appropriately restricted, observable and recoverable.

17. Decision

LLD-01-Q02 — Network Segmentation Model

Decision: Accepted for Northwind v1 conceptual network design, with subsequent detailed architecture established through LLD-06 and LLD-07.

Northwind v1 shall use the following security zones:

USER / SERVER / MGMT / PERIPHERAL / EXTERNAL-UNTRUSTED

The four internal security zones shall use VLANs 300, 200, 100 and 400 respectively.

FW01 shall provide the principal routed network security boundary.

JUMP01 shall provide the standard administrative entry point to internal RHEL servers.

Inter-zone communication shall use default-deny with explicitly permitted service flows.

Additional segmentation, including DMZ, dedicated backup and application-specific networks, shall only be introduced when supported by a defined business, security or technical requirement.

17.1 Decision rationale

This approach:

- Establishes meaningful trust boundaries.

- Limits unnecessary lateral movement.

- Separates ordinary service consumption from privileged administration.

- Supports the planned six-VM infrastructure.

- Avoids unnecessary VLAN complexity.

- Supports controlled firewall policies.

- Maintains independent hypervisor recovery access.

- Aligns with Northwind's recovery-based resilience model.

- Provides practical enterprise Linux administration experience.

18. Outstanding Actions

| Action | Required activity | Primary dependency |
| --- | --- | --- |
| LLD01-Q02-A001 | Confirm final IPv4 addressing and absence of home/VPN subnet overlap | LLD-06 |
| LLD01-Q02-A002 | Confirm FW01 external and internal interface configuration | LLD-01 / LLD-06 |
| LLD01-Q02-A003 | Complete required service connectivity and firewall rule specifications | LLD-06 |
| LLD01-Q02-A004 | Confirm CORE01 DNS/DHCP/time-service dependencies and bootstrap arrangements | LLD-02 / LLD-06 |
| LLD01-Q02-A005 | Confirm final guest VLAN assignments and VM interface specifications | LLD-01 |
| LLD01-Q02-A006 | Validate JUMP01 administrative access and SSH forwarding restrictions | LLD-07 |
| LLD01-Q02-A007 | Verify independent HYP01 management and console recovery | LLD-01 / LLD-07 |
| LLD01-Q02-A008 | Confirm IPv6 security and routing policy before enabling applicable connectivity | LLD-06 |
| LLD01-Q02-A009 | Record positive and negative network security acceptance test results | LLD-06 / LLD-07 |

These are proposed tracking identifiers for this revision, not evidence that Jira issues have already been created.

19. LLD Impact

| Design area | Impact |
| --- | --- |
| LLD-01 — Platform & Infrastructure Foundation | Virtual bridge design, VM interface connections, network dependencies and recovery access |
| LLD-02 — RHEL Operating System | NetworkManager, firewalld, SSH baseline and operating system validation |
| LLD-06 — Network Architecture | VLANs, IP addressing, routing, firewall policy and connectivity matrix |
| LLD-07 — Administrative Access | JUMP01, approved workstations, SSH ProxyJump and break-glass access |
| LLD-09 — Backup & Recovery | Backup communication, independent recovery storage and network restoration |
| LLD-12 — Security | Defence-in-depth, service restrictions and security validation |

The detailed LLD-06 design remains authoritative for the precise network implementation.

LLD-01-002 establishes the architectural reasoning and segmentation decisions.

20. Revision Impact Register

| Original design element | Classification | Revision outcome |
| --- | --- | --- |
| Five security zones | Retain | Original trust-boundary model preserved |
| Segmentation by trust rather than function | Retain | Remains governing design principle |
| Default-deny inter-zone policy | Retain | Explicitly associated with FW01 |
| USER Zone | Amend | VLAN 300 and proposed subnet added |
| SERVER Zone | Amend | VLAN 200 and current planned server estate added |
| MGMT Zone | Amend | VLAN 100 and JUMP01 placement confirmed |
| Proxmox management within MGMT concept | Clarify | HYP01 management remains independently accessible on the home network |
| PERIPHERAL Zone | Amend | VLAN 400 and proposed subnet added |
| EXTERNAL/UNTRUSTED Zone | Amend | Home-network and FW01 external connectivity clarified |
| Bastion-host model | Retain and expand | LLD-07 SSH ProxyJump and security decisions incorporated |
| Monitoring network placement | Resolve | OPS01 planned in SERVER |
| Backup VLAN | Retain deferral | No dedicated initial backup VLAN |
| Application VLANs | Retain deferral | Introduce only where justified |
| DMZ | Retain deferral | No initial requirement |
| VLAN IDs and addressing | Update | Previously deferred; now established or proposed through LLD-06 |
| Proxmox network architecture | Add | vmbr0, vmbr1, VLAN trunk and guest tagging |
| Independent recovery path | Add | HYP01 management and VM console recovery |
| Security validation | Expand | Positive and negative testing requirements |

21. Review and Approval

This revision incorporates the network and administrative access architecture established after the original LLD-01-002 working paper was written.

Before approval as the revised baseline, the following should be confirmed:

- Consistency with the final LLD-06 network specifications.

- Consistency with LLD-07 administrative access requirements.

- Actual Proxmox networking against Sprint 1 implementation evidence.

- Final FW01 and JUMP01 interface assignments.

- Outstanding IPv4/IPv6 and service-connectivity decisions.

Proposed disposition: Accept the revised conceptual architecture for technical review, with implementation-specific details remaining controlled through LLD-06, LLD-07 and the relevant implementation records.